. Programming document of AVDRV.DLL

DRAFT version 0.1

. Structure of ScannerAbility:

���������������������������������������������������������������������Ŀ

� Model                      �  8 Bytes   �Model name                 �

�                            �            �                           �

� Firmware Version           �  4 Bytes   �Firmware version           �

�                            �            �                           �

� Max Optical Resolution     �  WORD      �the maximum optical        �

�                            �            �resolution of this scanner �

�                            �            �                           �

� Maximum horizontal         �  WORD      �the maximum resolution     �

� resolution                 �            �of horizontal direction    �

�                            �            �                           �

� Maximum vertical           �  WORD      �the maximum resolution     �

� resolution                 �            �of vertical direction      �

�                            �            �                           �

� Image data width supported �  BYTE      �bit      bits per pixel    �

�                            �            � 7            1            �

�                            �            � 6            3            �

�                            �            � 5            4            �

�                            �            � 4            6            �

�                            �            � 3            8            �

�                            �            � 2            12           �

�                            �            � 1            18           �

�                            �            � 0            24           �

�                            �            �                           �

� Reserved                   �  BYTE      �                           �

�                            �            �                           �

� Image type supported       �  BYTE      �bit       image type       �

�                            �            � 7           B/W           �

�                            �            � 6         3-pass color    �

�                            �            � 5    1-pass color mode 1  �

�                            �            � 4    1-pass color mode 2  �

�                            �            � 3    1-pass color mode 3  �

�                            �            � others    reserved        �

�                            �            �                           �

� Scan method supported      �  BYTE      �bit       scan method      �

�                            �            � 7          flatbed        �

�                            �            � 6       roller type ADF   �

�                            �            � 5       flatbed type ADF  �

�                            �            � 4        roller W/O ADF   �

�                            �            � others    reserved        �

�                            �            �                           �

� Maximum frame width for    �  WORD      �maximum frame width of     �

� flatbed type               �            �flatbed type.              �

�                            �            �max width = 300 x inches   �

�                            �            �                           �

� Maximum frame length for   �  WORD      �maximum frame length of    �

� flatbed type               �            �flatbed type.              �

�                            �            �max length = 300 x inches  �

�                            �            �                           �

� Maximum frame width for    �  WORD      �max width = 300 x inches   �

� ADF/Roller type            �            �                           �

�                            �            �                           �

� Maximum frame length for   �  WORD      �max length = 300 x inches  �

� ADF/Roller type            �            �                           �

�                            �            �                           �

� Reserved                   �  BYTE      �                           �

�                            �            �                           �

� Number of build-in         �  BYTE      �                           �

� halftone pattern           �            �                           �

�                            �            �                           �

� Support Highlight/Shadow   �  BYTE      �0 - No support             �

�                            �            �1 - Support                �

�                            �            �                           �

� Support Invert             �  BYTE      �0 - No support             �

�                            �            �1 - Support                �

�                            �            �                           �

� Reserved                   �  10 Bytes  �                           �

�����������������������������������������������������������������������

. Structure of ScannerParameter:

���������������������������������������������������������������������Ŀ

� Left coordinate of scanning�  WORD      � Value = 300 X inches      �

� frame                      �            �                           �

�                            �            �                           �

� Top coordinate of scanning �  WORD      � Value = 300 X inches      �

� frame                      �            �                           �

�                            �            �                           �

� Width of scanning frame    �  WORD      � Value = 300 X inches      �

�                            �            �                           �

�                            �            �                           �

� Length of scanning frame   �  WORD      � Value = 300 X inches      �

�                            �            �                           �

� Pixel number of horizontal �  WORD      � Actural pixels per line   �

�                            �            �                           �

� Line number of vertical    �  WORD      � Actural lines per frame   �

���������������������������������������������������������������������Ĵ

�                                                                     �

� Left, Top, Width and Length is used to define the LOGICAL           �

� coordinate of scanning area. You don't need to care what is the     �

� resolution of scanner. For example, if you want to scan an image on �

� (1.5", 2.3") and width and length is (3.4", 5.2") then:             �

�                                                                     �

� Left = 1.5*300 = 450, Top = 2.3*300 = 690                           �

� Width = 3.4*300 = 1020, Length = 5.2*300 = 1560                     �

�                                                                     �

� Pixel and Line number is used to define the actual size of image    �

� that you want to scan. For example, if you want to scan an image    �

� width and length is (3.4", 5.2"), and resolution is 200 dpi, then:  �

�                                                                     �

� Pixels = 3.4*200 = 680                                              �

� Lines = 1040                                                        �

�                                                                     �

� If the image that you scan is single bit image, like lineart,       �

� halftone, error diffusion, the pixel number should be the multiple  �

� of 32. And if the image is gray or color, the pixel number should   �

� be the multiple of 4.                                               �

�                                                                     �

���������������������������������������������������������������������Ĵ

�                            �            �                           �

� Scanning mode              �  BYTE      � 0 - Lineart               �

�                            �            � 1 - Halfton               �

�                            �            � 2 - Gray                  �

�                            �            � 3 - 3-pass color          �

�                            �            � 4 - 1-pass color          �

�                            �            � Other - reserved          �

�                            �            �                           �

�                            �            � Not all scanner support   �

�                            �            � above "Scanning Mode"     �

�                            �            �                           �

� Scanning method            �  BYTE      � 0 - Flatbed mode          �

�                            �            � 1 - ADF mode              �

�                            �            � 2 - Transparency mode     �

�                            �            � Other - reserved          �

�                            �            �                           �

� Bits per pixel             �  BYTE      � 1 - available for Lineart/�

�                            �            �     Halftone mode         �

�                            �            � 8 - available for gray/   �

�                            �            �     color mode            �

�                            �            � Other - reserved          �

�                            �            �                           �

�                            �            � Note: Not all scanner     �

�                            �            �       support above       �

�                            �            �       "Bits per pixel"    �

�                            �            �                           �

� Scanning speed             �  BYTE      � range: 1 to 15            �

�                            �            � speed 1 is fastest        �

�                            �            � speed 15 is the slowest   �

�                            �            � speed N = speed 1 / N     �

�                            �            �                           �

� Contrast                   �  char      � range: -128 to 127        �

�                            �            �                           �

� Brightness                 �  char      � range: -128 to 127        �

�                            �            �                           �

� Halftone pattern number    �  BYTE      � range: 1 to 7             �

�                            �            �                           �

� Highlight                  �  BYTE      � range: 1 to 255           �

�                            �            � default = 255             �

�                            �            �                           �

� Shadow                     �  BYTE      � range: 0 to 254           �

�                            �            � default = 0               �

�                            �            �                           �

�                            �            � Shadow should be always   �

�                            �            � smaller than Highlight    �

�                            �            �                           �

� Color filter selection     �  BYTE      � 0 - Normal                �

�                            �            � 1 - Red                   �

�                            �            � 2 - Green                 �

�                            �            � 3 - Blue                  �

�                            �            �                           �

� Invert Image               �  BYTE      � 0 - Normal                �

�                            �            � 1 - Invert Image          �

�                            �            �                           �

� Read Status                �  BYTE      � 0 - Don't Read scan status�

�                            �            � 1 - Read scan status      �

�                            �            �                           �

�                            �            � Currently, do not use 1   �

�                            �            �                           �

� Reserved                   �  10 Bytes  � Must be 0 in order to     �

�                            �            � be compatible with further�

�                            �            � scanner or driver         �

�                            �            �                           �

�����������������������������������������������������������������������

. Function description:

1. BOOL InitializeScanner(void);

Before calling other function, it is necessary to call this function first.

It is used to initialize scanner. If the return is failed, it means the

driver can not find the scanner.

2. BOOL ResetScanner(LPSCANPARAMETER pParam);

Programmer can use this function to reset the scanning paramerter and get

the default scanning parameter.

Return FALSE means the driver can not reset or find the scanner.

3. BOOL GetScannerAbility(LPSCANNERABILITY pAbility);

Programmer can get the scanner ability by using this function. It will

return which ability that this scanner can provide.

4. BOOL SetScanParameter(LPSCANPARAMETER pParam);

Before do StartScan(), programmer should use this function to tell scanner

what is the scanning parameter that you want.

5. BOOL GetADFStatus(LPBYTE pStatus);

Use the check the ADF status.

pStatus: 0 - No ADF or no paper on ADF

1 - ADF exist and has paper on ADF.

You can use this function to check the ADF status before scanning, or used

to do multiple page scanning.

6. BOOL StartScan(VOID);

Used to tell scanner to start scanning. After calling this function, you can

just call ReadScan to read image from scanner, or StopScan to stop scanning.

You can not call any other function call.

7. BOOL ReadScan(LPBYTE pStatus, LPBYTE pBuffer, WORD Count);

Used to get image from scanner, the driver will return the RAW data from

scanner to application.

pStatus - The scanner status, currently, do not support.

pBuffer - The return image data buffer

Count - The size of image that you want to get.

If you can not read whole image once, you can use a loop to call this

function to get whole image.

8. BOOL StopScan(VOID);

Used to stop or cancel scanning.

9. BOOL DoCalibration(BYTE Type, BYTE Mode, BYTE Bits);

Ask scanner to do calibration, only, AV6240, AV100C, AV3C, AV25 support

this function.

10. BOOL DoPaperFeed(void);

Ask scanner to feed paper, only AV100C, AV3C, AV25 support this function.

11. BOOL SetGammaTable(LPBYTE GammaTable, BYTE Color, WORD Size);

Download gamma table to scanner, only AV6240, AV100C, AV3C, AV25 support

this function.

GammaTable - The gamma table that you want to download.

Color - Which color gamma that you want to download

0 - Master

1 - Red

2 - Green

3 - Blue

Size - The size of Gamma Table, currently, only 256 support.

AV6240, AV100C, AV3C, AV25 do not support Brightness, Contrast, Highlight,

Shadow. You need to use SetGammaTable to do that. Following is the sample

code of build up GammaTable:

/* Input: GammaTable - A 256 byte GammaTable

Brightness: -127 - 127

Contrast: -127 - 127

Highlight: 1 - 255

Shadow: 0 - 254              * Shadow should be smaller than Highlight

Gamma: 10 - 999, 0.1 - 99.9

*/

void CalGammaTable(LPBYTE GammaTable, int Brightness, int Contrast,

BYTE Highlight, BYTE Shadow, float Gamma)

{

int i, j;

int Start, End;

long ii, jj;

float fPixel;

for (i=0; i<255; i++)

GammaTable[i] = i;

Start  = Contrast + ((WORD)Shadow*(255-Contrast*2)/255);

End = Contrast + ((255-Highlight)*(255-Contrast*2)/255);

if ((Start>=0) && (End>=0))

{

for (i=0; i<Start; i++)

GammaTable[i]=0;

for (i=(255-End); i<256; i++)

GammaTable[i]=255;

for (i=Start, jj=0; i<(255-End); i++, jj++)

GammaTable[i] = (BYTE)(jj*255/(255-Start-End));

}

else if ((Start>=0) && (End<0))

{

for (i=0; i<Start; i++)

GammaTable[i]=0;

for (i=Start, jj=0; i<256; i++, jj++)

GammaTable[i] = (BYTE)(jj*255/(255-Start-End));

}

else if ((Start<0) && (End>=0))

{

for (i=(255-End); i<256; i++)

GammaTable[i]=255;

for (i=0, jj=0; i<(255-End); i++, jj++)

GammaTable[i] = (BYTE)(jj*255/(255-Start-End)) - Start;

}

else         //Start < 0 and End < 0

{

for (i=0, jj=0; i<256; i++, jj++)

GammaTable[i] = (BYTE)(jj*255/(255-Start-End)) - Start;

}

if (Brightness!=0)

{

for (i=0; i<256; i++)

{

j = GammaTable[i];

j += Brightness;

if (j<0) j = 0;

if (j>255) j = 255;

GammaTable[i] = (BYTE)j;

}

}

if (Gamma == 0)

Gamma = 1;

else

Gamma = Gamma/100;

Gamma = 1/Gamma;

for (i=0; i<256; i++)

{

j = GammaTable[i];

jj = j;

fPixel = (float)jj;

fPixel = fPixel / 255;

fPixel = pow(fPixel, Gamma);

fPixel = fPixel * 255;

j = (BYTE)(fPixel+0.5);

GammaTable[i] = j;

}

}

. General function call procedure without ADF:

InitializeScanner();

GetSannerAbility();

ResetScanner();

SetScanParameter();

StartScan();

while (not read whole image)

ReadScan();                //Read one block of image from scanner

StopScan();

. General function call procedure with ADF:

InitializeScanner();

GetSannerAbility();

ResetScanner();

NextPage:

GetADFStatus();

if (no paper)

return;

SetScanParameter();

StartScan();

while (not read whole image)

ReadScan();                //Read one block of image from scanner

StopScan();

goto NextPage;