Chapter 0. Released notes.

Chapter 1. Driver installation and function description

Chapter 2. Notification (Very important!)

Chapter 0. Released notes

In this package, there have following files:

1. AVDRV.SYS - The device driver of AVision scanner series.

2. AVDRV.DOC - This file.

3. SAMPLE.C - A sample program that using C language.

4. SAMPLE.EXE - The execution code of SAMPLE.C.

Chapter 1. Driver installation and function description

1.0 How to Install AVision Scanner Device Driver

In order to use AVision Scanner Device Driver, you have to add

following line in your CONFIG.SYS.

DEVICE = AVDRV.SYS /N

N -> Set the scanner interface card address.

Each value of N is corresponding to a scanner address.

DIP Switch: SW1 SW2 SW3 SW4   port address  N

0   0   1   0  :      220      1    <= Default

0   1   0   1  :      290      2

1   0   1   0  :      320      3

1   1   0   1  :      390      4

0   0   0   0  :      200      5

0   0   0   1  :      210      6

0   0   1   1  :      230      7

0   1   0   0  :      280      8

0   1   1   0  :      2A0      9

0   1   1   1  :      2B0      10

1   0   0   0  :      300      11

1   0   0   1  :      310      12

1   0   1   1  :      330      13

1   1   0   0  :      380      14

1   1   1   0  :      3A0      15

1   1   1   1  :      3B0      16

1 - OFF

0 - ON

Sample: Use 4th I/O address

DEVICE = C:\DRIVER\AVDRV.SYS /4

1.1 How to Use AVision Scanner Device Driver

For AVision Scanner Device Driver is PURE a Character Device Driver,

so you should use DOS OPEN to open the device driver, DOS WRITE to

send command/function and DOS READ to receive return message, IOCTL READ

to get image from scanner and DOS CLOSE to close/finish device driver.

A. Open driver

AH = 3DH

AL  =  2

DS:DX -> A pointer point to scanner driver name.

The driver name should be "AV800DRV"

INT = 21H

Return: AX - the File handle number

The file attribute should be as raw mode not cook mode.

Assembly Sample:

DeviceName        db      'AV800DRV',0

mov     dx,offset DeviceName       ;The device driver name

mov     ah,3dh

mov     al,2

mov     cx,0

int     21h

mov     scanner_handle,ax

mov     ax,4400h                ;Change file attribute to raw mode

mov     bx,scanner_handle

int     21h

mov     dh,0

or      dl,20h

mov     ax,4401h

int     21h

C Sample

// DeviceName is the device driver name

static char DeviceName[] = "AV800DRV";

if (( hScanner = open( &DeviceName, O_RDWR)) == -1)

{

printf("Device Driver not ready\n");

return 0;

}

_asm {                    // Change file attribute to raw mode

mov     ax,4400h

mov     bx,ScannerHandle

int     21h

mov     ax,4401h

mov     dh,0

or      dl,20h

int     21h

}

B. WRITE function

AH = 40H

BX = File handle

CX = Number of bytes to send to scanner driver

DS:DX -> Pointer that point to transfer block

INT = 21H

Assembly Sample A.

mov     ax,seg CommandBlock

mov     ds,ax

mov     dx,offset CommandBlock

mov     ah,40h

mov     bx,scanner_handle

mov     cx,1

int     21h

C Sample

if (( res = write(hScanner, &CommandBlock, sizeof(CommandBlock))) == -1)

{

printf("Scanner error!\n");

close(hScanner);

return 0;

}

B. READ function

AH = 3FH

BX = File Handle returned by open scanner driver

CX = Number of bytes to receive from scanner.

DS:DX -> Pointer that point to transfer block

INT = 21H

Assembly Sample

mov     ax,seg BufSize

mov     ds,ax

mov     dx,offset BufSize

mov     ah,3fh

mov     bx,scanner_handle

mov     cx,BufSize

int     21h

Sample B.

if (( res = read(hScanner, &Buf, sizeof(Buf))) == -1)

{

close(hScanner);

return 0;

}

D. IOCTL READ function

AH = 44H

AL = 02H

BX = File Handle returned by open scanner driver

CX = Number of bytes to receive from scanner.

DS:DX -> Pointer that point to image buffer

INT = 21H

Assembly Sample

mov     ax,seg BufSize

mov     ds,ax

mov     dx,offset BufSize

mov     ah,44h

mov     al,2

mov     bx,scanner_handle

mov     cx,BufSize

int     21h

C Sample

_asm {

mov     ax,4402h      // IOCTL Read

mov     bx,ScannerHandle

mov     cx,BufSize

mov     dx,BufOffset

int     21h

}

There are a more detail sample in SAMPLE.C file.

E. Close driver

AH = 3EH

BX = File Handle returned by open scanner driver

INT = 21H

Assembly Sample

mov     ah,3EH

mov     bx,scanner_handle

int     21h

C Sample

close(hScanner);

1.2 Function description

Function 0 - Initialize scanner

Write:

Parameter Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Function No                �  BYTE      �   0    �                  �

�����������������������������������������������������������������������

Read:

Return Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Return Status              �  BYTE      �   0    �  Success         �

�                            �            �   FE   �  Time out        �

�                            �            �   FF   �  Fail            �

�                            �            � others �  Reserved        �

�����������������������������������������������������������������������

Note: Programmer should call this function before calling any other

functions. After calling this function, scanner will do self-test,

reset all scanner status and clear all windows.

This function will need about 5 seconds to complete, so if you

just want to reset parameter, you have better to call function 2

to do reset.

Function 1 - Get scanner ability

Write:

Parameter Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Function No                �  BYTE      �   1    �                  �

�����������������������������������������������������������������������

Read:

Parameter Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Return Status              �  BYTE      �   0    �  Success         �

�                            �            �   FE   �  Time out        �

�                            �            �   FF   �  Fail            �

�                            �            � others �  Reserved        �

���������������������������������������������������������������������Ĵ

� Model                      �  8 Bytes   �Model name                 �

�                            �            �                           �

� Firmware Version           �  4 Bytes   �Firmware version           �

�                            �            �                           �

� Max Optical Resolution     �  WORD      �the maximum optical        �

�                            �            �resolution of this scanner �

�                            �            �                           �

� Maximum horizontal         �  WORD      �the maximum resolution     �

� resolution                 �            �of horizontal direction    �

�                            �            �                           �

� Maximum vertical           �  WORD      �the maximum resolution     �

� resolution                 �            �of vertical direction      �

�                            �            �                           �

� Image data width supported �  BYTE      �bit      bits per pixel    �

�                            �            � 7            1            �

�                            �            � 6            3            �

�                            �            � 5            4            �

�                            �            � 4            6            �

�                            �            � 3            8            �

�                            �            � 2            12           �

�                            �            � 1            18           �

�                            �            � 0            24           �

�                            �            �                           �

� Reserved                   �  BYTE      �                           �

�                            �            �                           �

� Image type supported       �  BYTE      �bit       image type       �

�                            �            � 7           B/W           �

�                            �            � 6         3-pass color    �

�                            �            � 5    1-pass color mode 1  �

�                            �            � 4    1-pass color mode 2  �

�                            �            � 3    1-pass color mode 3  �

�                            �            � others    reserved        �

�                            �            �                           �

� Scan method supported      �  BYTE      �bit       scan method      �

�                            �            � 7          flatbed        �

�                            �            � 6       roller type ADF   �

�                            �            � 5       flatbed type ADF  �

�                            �            � 4        roller W/O ADF   �

�                            �            � others    reserved        �

�                            �            �                           �

� Maximum frame width for    �  WORD      �maximum frame width of     �

� flatbed type               �            �flatbed type.              �

�                            �            �max width = 300 x inches   �

�                            �            �                           �

� Maximum frame length for   �  WORD      �maximum frame length of    �

� flatbed type               �            �flatbed type.              �

�                            �            �max length = 300 x inches  �

�                            �            �                           �

� Maximum frame width for    �  WORD      �max width = 300 x inches   �

� ADF/Roller type            �            �                           �

�                            �            �                           �

� Maximum frame length for   �  WORD      �max length = 300 x inches  �

� ADF/Roller type            �            �                           �

�                            �            �                           �

� Reserved                   �  BYTE      �                           �

�                            �            �                           �

� Number of build-in         �  BYTE      �                           �

� halftone pattern           �            �                           �

�                            �            �                           �

� Support Highlight/Shadow   �  BYTE      �0 - No support             �

�                            �            �1 - Support                �

�                            �            �                           �

� Reserved                   �  10 Bytes  �                           �

�����������������������������������������������������������������������

Function 2 - Reset scanner/Get scanner default major parameter

Write:

Parameter Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Function No                �  BYTE      �   2    �                  �

�����������������������������������������������������������������������

Read:

return Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Return Status              �  BYTE      �   0    �  Success         �

�                            �            �   FE   �  Time out        �

�                            �            �   FF   �  Fail            �

�                            �            � others �  Reserved        �

���������������������������������������������������������������������Ĵ

� Left coordinate of scanning�  WORD      �    0                      �

� frame                      �            �                           �

� Top coordinate of scanning �  WORD      �    0                      �

� frame                      �            �                           �

� Width of scanning frame    �  WORD      � 2550                      �

� Length of scanning frame   �  WORD      � 3300                      �

� Pixel number of horizontal �  WORD      � 2550                      �

� direction                  �            �                           �

� Line number of vertical    �  WORD      � 3300                      �

� direction                  �            �                           �

� Scanning mode              �  BYTE      �  0                        �

� Scanning method            �  BYTE      �  0                        �

� Bits per pixel             �  BYTE      �  1                        �

� Scanning speed             �  BYTE      �  1                        �

� Contrast                   �  BYTE      �  0                        �

� Brightness                 �  BYTE      �  0                        �

� Halftone pattern number    �  BYTE      �  5                        �

� Highlight value            �  BYTE      � 255                       �

� Shadow value               �  BYTE      �  0                        �

� Color filter selection     �  BYTE      �  0                        �

� Invert Image               �  BYTE      �  0                        �

� Reserved                   �  9 Bytes   �  0                        �

�����������������������������������������������������������������������

Note: After calling this function, Device Driver will reset it's own

parameter. Including reset parameter to default, clear all windows.

Function 3 - Set scanner major parameter

Write:

Parameter Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Function No                �  BYTE      �   3    �                  �

���������������������������������������������������������������������Ĵ

� Left coordinate of scanning�  WORD      �                           �

� frame                      �            �                           �

�                            �            �                           �

� Top coordinate of scanning �  WORD      �                           �

� frame                      �            �                           �

�                            �            �                           �

� Width of scanning frame    �  WORD      �                           �

�                            �            �                           �

�                            �            �                           �

� Length of scanning frame   �  WORD      �                           �

�                            �            �                           �

� Pixel number of horizontal �  WORD      �                           �

�                            �            �                           �

� Line number of vertical    �  WORD      �                           �

�                            �            �                           �

� Scanning mode              �  BYTE      � 0 - Lineart               �

�                            �            � 1 - Halfton               �

�                            �            � 2 - Gray                  �

�                            �            � 3 - 3-pass color          �

�                            �            � 4 - 1-pass color mode 1   �

�                            �            � 5 - 1-pass color mode 2   �

�                            �            � 6 - 1-pass color mode 3   �

�                            �            � Other - reserved          �

�                            �            �                           �

�                            �            � mode 1 - the color line   �

�                            �            � sequence is RGB 1st line  �

�                            �            � -> RGB 2nd line           �

�                            �            �                           �

�                            �            � mode 2 - the color line   �

�                            �            � sequence is R 1st line -> �

�                            �            � G 1st line -> B 1st line  �

�                            �            � R 2nd line -> G 2nd line  �

�                            �            � B 2nd line                �

�                            �            �                           �

�                            �            � mode 3 - the color line   �

�                            �            � sequence is not continuous�

�                            �            � not yet define            �

�                            �            �                           �

�                            �            � Not all scanner support   �

�                            �            � above "Scanning Mode"     �

�                            �            �                           �

� Scanning method            �  BYTE      � 0 - Flatbed mode          �

�                            �            � 1 - ADF mode              �

�                            �            � 2 - Transparency mode     �

�                            �            � Other - reserved          �

�                            �            �                           �

� Bits per pixel             �  BYTE      � 1 - available for Lineart/�

�                            �            �     Halftone mode         �

�                            �            � 8 - available for gray/   �

�                            �            �     color mode            �

�                            �            � Other - reserved          �

�                            �            �                           �

�                            �            � Note: Not all scanner     �

�                            �            �       support above       �

�                            �            �       "Bits per pixel"    �

�                            �            �                           �

� Scanning speed             �  BYTE      � range: 1 to 15            �

�                            �            � speed 1 is fastest        �

�                            �            � speed 15 is the slowest   �

�                            �            � speed N = speed 1 / N     �

�                            �            �                           �

� Contrast                   �  char      � range: -128 to 127        �

�                            �            �                           �

� Brightness                 �  char      � range: -128 to 127        �

�                            �            �                           �

� Halftone pattern number    �  BYTE      �                           �

�                            �            �                           �

� Highlight                  �  BYTE      � range: 1 to 255           �

�                            �            � default = 255             �

�                            �            �                           �

� Shadow                     �  BYTE      � range: 0 to 254           �

�                            �            � default = 0               �

�                            �            �                           �

�                            �            � Shadow should be always   �

�                            �            � smaller than Highlight    �

�                            �            �                           �

� Color filter selection     �  BYTE      � 0 - Normal                �

�                            �            � 1 - Red                   �

�                            �            � 2 - Green                 �

�                            �            � 3 - Blue                  �

�                            �            �                           �

� Invert Image               �  BYTE      � 0 - Normal                �

�                            �            � 1 - Invert Image          �

�                            �            �                           �

� Reserved                   �  9 Bytes   � Must be 0 in order to     �

�                            �            � be compatible with further�

�                            �            � scanner or driver         �

�                            �            �                           �

�����������������������������������������������������������������������

Read:

Return Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Return Status              �  BYTE      �   0    �  Success         �

�                            �            �   FD   �  Parameter error �

�                            �            �   FE   �  Time out        �

�                            �            �   FF   �  Fail            �

�                            �            � others �  Reserved        �

�����������������������������������������������������������������������

Function 4 - Reserved

Function 5 - Inquiry scanner/ADF status

Write:

Parameter Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Function No                �  BYTE      �   5    �                  �

�����������������������������������������������������������������������

Read:

Return Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Return Status              �  BYTE      �   0    �  Fine            �

�                            �            �   FE   �  Time out        �

�                            �            �   FF   �  Fail            �

�                            �            � others �  Reserved        �

���������������������������������������������������������������������Ĵ

� Scanner status             �  BYTE      � 0 - Ready                 �

�                            �            � 1 - Busy                  �

�                            �            � Others - Error            �

�                            �            �                           �

� ADF status                 �  BYTE      � bit       description     �

�                            �            �  7         ADF exist      �

�                            �            �  6         ADF ready      �

�                            �            �  5       ADF have paper   �

�                            �            �  4        ADF paper jam   �

�                            �            � others      reserved      �

�                            �            �                           �

� Reserved                   �  10 Bytes  � 0                         �

�                            �            �                           �

�����������������������������������������������������������������������

Function 6 - Start scan

Write:

Parameter Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Function No                �  BYTE      �   6    �                  �

�����������������������������������������������������������������������

Read:

Return Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Return Status              �  BYTE      �   0    �  Success         �

�                            �            �   FE   �  Time out        �

�                            �            �   FF   �  Fail            �

�                            �            � others �  Reserved        �

�����������������������������������������������������������������������

Note: After calling function 6, programmer must use IOCTL Read to get

image data.

Function 7 - Stop scan

Write:

Parameter Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Function No                �  BYTE      �   7    �                  �

�����������������������������������������������������������������������

Read:

Return Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Return Status              �  BYTE      �   0    �  Success         �

�                            �            �   FE   �  Time out        �

�                            �            �   FF   �  Fail            �

�                            �            � others �  Reserved        �

�����������������������������������������������������������������������

Function 20 - Do transparency calibration

Write:

Parameter Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Function No                �  BYTE      �   20   �                  �

� Scanning Mode              �  BYTE      �    0   � Lineart/Halftone �

�                            �            �        � Gray             �

�                            �            �    1   � Color            �

� Bits Per Pixel             �  BYTE      �    1   � Lineart/Halftone �

�                            �            �    8   � Gray/Color       �

�����������������������������������������������������������������������

Read:

Return Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Return Status              �  BYTE      �   0    �  Success         �

�                            �            �   FE   �  Time out        �

�                            �            �   FF   �  Fail            �

�                            �            � others �  Reserved        �

�����������������������������������������������������������������������

Function 21 - Do AV6240 color calibration

Write:

Parameter Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Function No                �  BYTE      �   21   �                  �

�����������������������������������������������������������������������

Read:

Return Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Return Status              �  BYTE      �   0    �  Success         �

�                            �            �   FE   �  Time out        �

�                            �            �   FF   �  Fail            �

�                            �            � others �  Reserved        �

�����������������������������������������������������������������������

Function 30 - Download gamma table (AV6240 only)

Write:

Parameter Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Function No                �  BYTE      �   30   �                  �

� Gamma data                 �  4096 BYTE �        �                  �

�����������������������������������������������������������������������

Read:

Return Block :

���������������������������������������������������������������������Ŀ

� Item                       � Data width � Value  �  Description     �

���������������������������������������������������������������������͵

� Return Status              �  BYTE      �   0    �  Success         �

�                            �            �   FE   �  Time out        �

�                            �            �   FF   �  Fail            �

�                            �            � others �  Reserved        �

�����������������������������������������������������������������������

Chapter 2 Notifications:

1.0 Nots for programmer that use Microsoft C

1. Microsoft C can't open a file (or Device Driver) as binary/raw mode, so

you should change the file (or Device Driver) attribute by yourself.

Following is an example (use MSC 6.0 or later):

// Open Scanner Device Driver

if (( ScannerHandle = open( DeviceName, O_BINARY|O_RDWR)) == -1)

return FALSE;

_asm {                    // Change file attribute to raw mode

mov     ax,4400h

mov     bx,ScannerHandle

int     21h

mov     ax,4401h

mov     dh,0

or      dl,20h

int     21h

}

2. While compiling your program, you should add /Zp option.

When using AVDRV.SYS, it is necessary to use BYTE oriented not

WORD. But Microsoft C compiler use WORD oriented as default, so

you should add /Zp to change to BYTE oriented.

Following is an example (use MSC 6.0 or later):

cl /Zp XXXX.C

1.1 Notes for Windows 3.x programmer

1. In most cases, you can use AVDRV.SYS in Windows 3.x as same as

that you use in DOS.

Following are some tips about using AVDRV.SYS in Windows 3.x:

a. Use static array as image buffer.

static BYTE ImageBuffer[8192];

b. Use in-line assembly while use IOCTL READ to get image.

static  int   ReadImage( char * pBuffer, int nCnt )

{

int     ret = 0;

_asm {

mov     ah,44h          //IOCTL Read

mov     al,2

mov     bx,hScanner

mov     cx,nCnt

mov     dx,pBuffer

int     21h

jnc     No_Err

}

ret = -1;

No_Err:

return ret;

}

AVDRV.SYS should work fine in Windows 3.x, most of our application

drivers are using AVDRV.SYS, like Pixel Translation PXW driver,

Picture Publisher driver, PhotoStyler Plug-in driver.